Updated: 7/18/19

Materials:

- Gloves

- Culture samples

- 90% Acetone (fridge)

- Deionized water

- Micro-pipette (1 mL max)

- Micro-pipette (20-200µg)

- Refrigerated Centrifuge

- Bead-mill

- Refrigerator

- Photo Spectrometer

- Fume Hood

- Minimize lights in laboratory as much as possible to decrease light damage to pigments.

- Set the refrigerated centrifuge to 4℃.

- Allow samples to thaw in the dark. Keep on ice after thawing.

- Ce54ntrifuge for 20 min at max RPM (14.8) and 4℃ .

- Remove samples from centrifuge and very carefully put them on ice.

- Using the 1mL maximum micro-pipette, carefully remove supernatant without disturbing pellet. Remove any remaining supernatant with the 20-200µg micropipette.

- Add small amount of beads to tubes.

- Add 1mL of 90% acetone to each tube once the supernatant has been drained.

- Run the samples in the bead mill at 6.00 m/s for 2 minutes.

- Cover each sample with foil (should be in the incubation box) and incubate in darkness for 24 hours in the fridge.

- After 24 hours, centrifuge the samples again for 3 minute at max RPM (14.8), 4℃.

- Plate samples in triplicate (200µg) and read absorbance at 630nm and 663 nm, use 90% acetone as blank. See picture for plate layout.

- Clear lidded assay plates (REF#:3370, https://ecatalog.corning.com/life-sciences/b2c/US/en/Permeable-Supports/Inserts/Corning%C2%AE-96-well-Clear-Polystyrene-Microplates/p/3370?clear=true) or black assay plates with clear bottoms can be used, but plate type must be consistent within a project. Plates can be rinsed and reused between readings.

- The formula for ChlA concentration=(13.31*Blk 663 average)-(0.27*Blk 630 average). The formula for ChlC2 concentration=(-8.37*Blk 663 average)+(51.72*Blk 630 average).